### Questions:

- At a temperature of 27°C (common temperature for components in an enclosed operating system), determine the thermal voltage VT.

- Prove that "The higher the current through a diode, the lower is the DC resistance level."

- Compare conductors, insulators, and semiconductors using band theory.

### Answers:

- Thermal Voltage VT Calculation:

The thermal voltage VT at a given temperature is given by the equation:

VT=kTq

Where:

- k is Boltzmann's constant 1.38×10−23 J/K,

- q is the charge of an electron 1.6×10−19 C,

- T is the absolute temperature in Kelvin.

Converting 27°C to Kelvin:

T=27+273.15=300.15 K

Now, calculating the thermal voltage:

VT=1.38×10−23×300.151.6×10−19≈0.0259 V or 25.9 mV

- Proving that Higher Current Results in Lower DC Resistance in a Diode:

The DC resistance of a diode is defined as:

Rdc=VI

Where V is the voltage across the diode and I is the current through it.

From the diode equation:

I=ISeVnVT−1

For significant forward bias, where V is large enough, the exponential term dominates, and the current approximately becomes:

I≈ISeVnVT

As the current I increases exponentially with voltage, the ratio VI, which represents the DC resistance Rdc, decreases. This proves that as the current through the diode increases, the DC resistance decreases.

- Comparison of Conductors, Insulators, and Semiconductors Using Band Theory:

In band theory, materials are classified based on the energy gap between their valence and conduction bands:

- Conductors:

  - In conductors, the valence and conduction bands either overlap or the conduction band is partially filled. As a result, electrons can move freely between these bands, allowing for high electrical conductivity.

  - There is effectively no energy gap, making it easy for electrons to move and conduct electricity.

- Insulators:

  - In insulators, there is a large energy gap (typically greater than 3 eV) between the valence and conduction bands. Electrons in the valence band require a large amount of energy to move to the conduction band, which makes insulators poor conductors of electricity.

- Semiconductors:

  - Semiconductors have a moderate energy gap (around 1 eV). At low temperatures, they behave like insulators because there aren't enough electrons with sufficient energy to cross the band gap. However, at higher temperatures or when doped, electrons can gain enough energy to jump to the conduction band, allowing the material to conduct electricity.

#### Band Gap Summary:

- Conductors: No band gap.

- Insulators: Large band gap (> 3 eV).

- Semiconductors: Moderate band gap (~1 eV).

This explains why conductors readily allow current flow, insulators resist it, and semiconductors can act as either depending on conditions.