### Questions:

a) Define electric field. Prove that electric field E exists at any point in space, and we can think of that point as the site of stored energy in amount, per unit volume, of 12ϵ0E2.

b) The following figure shows a dielectric slab of thickness b and dielectric constant k placed between the plates of a parallel-plate capacitor of plate area A and separation d. A potential difference V0 is applied with no dielectric present. The battery is then disconnected and the dielectric slab inserted. Assume A=100 cm2, d=1.0 cm, b=0.50 cm, k=7.0, and V0=100 volts.

(i) Calculate the capacitance C0 before the slab is inserted.
(ii) Calculate the free charge q.
(iii) Calculate the electric field strength in the gap.
(iv) Calculate the electric field strength in the dielectric.
(v) Calculate the potential difference between the plates.
(vi) Calculate the capacitance with the slab in place.

### Answers:

a) Electric Field Definition: The electric field is a vector field that represents the force per unit charge exerted on a test charge placed in the field. Mathematically:

E=Fq

where F is the force and q is the charge.

Energy Stored in an Electric Field: The energy density u stored in an electric field at any point in space is given by:

u=12ϵ0E2

where ϵ0 is the permittivity of free space and E is the electric field strength. This can be derived from the total electrostatic potential energy stored in a capacitor and expressed in terms of the electric field.

b) Given values:
A=100 cm2=1.0×10−2 m2,
d=1.0 cm=1.0×10−2 m,
b=0.50 cm=5.0×10−3 m,
k=7.0,
V0=100 volts.

(i) Capacitance C0 before the slab is inserted: The capacitance of a parallel-plate capacitor without a dielectric is:

C0=ϵ0Ad

where ϵ0=8.85×10−12 F/m.

Substitute the values:

C0=8.85×10−12×1.0×10−21.0×10−2=8.85×10−12 F=8.85 pF

(ii) Free charge q: The charge on the plates is given by:

q=C0V0

Substitute the values:

q=8.85×10−12×100=8.85×10−10 C

(iii) Electric field strength in the gap: The electric field in the gap is given by:

Egap=V0d

Substitute the values:

Egap=1001.0×10−2=10,000 V/m

(iv) Electric field strength in the dielectric: The electric field inside a dielectric is reduced by a factor of k (the dielectric constant):

Edielectric=Egapk

Substitute the values:

Edielectric=10,0007≈1428.57 V/m

(v) Potential difference between the plates: The total potential difference across the plates can be found by considering the potential drop in both the dielectric and the gap:

V=Edielectric⋅b+Egap⋅d−b

Substitute the values:

V=1428.57×5.0×10−3+10,000×5.0×10−3=7.14+50=57.14 volts

(vi) Capacitance with the slab in place: The capacitance with a dielectric is given by:

C=ϵ0Ad−b+bk

Substitute the values:

C=8.85×10−12×1.0×10−21.0×10−2−5.0×10−3+5.0×10−37≈1.09×10−11 F=10.9 pF