Problem 1: Declaring Input and Output in 8086 Processor

Objective: To understand and demonstrate the fundamental assembly language instructions and DOS interrupts (INT 21H) required to display text (output) to the screen and read a character (input) from the keyboard in an 8086 microprocessor environment.

Source Code:

.MODEL SMALL

.STACK 100H

.DATA

MSG1 DB 'Enter a character: $'

MSG2 DB 0DH,0AH,'You entered: $'

.CODE

MAIN PROC

MOV AX, @DATA

MOV DS, AX

MOV DX, OFFSET MSG1

MOV AH, 09H

INT 21H

MOV AH, 01H

INT 21H

MOV BL, AL

MOV DX, OFFSET MSG2

MOV AH, 09H

INT 21H

MOV DL, BL

MOV AH, 02H

INT 21H

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

Output

Figure 1: Taking Input and Showing Output

Problem 2: Declaring Variables in 8086 Processor

Objective: To learn the syntax and directives used to declare variables (data) of different sizes in the .DATA segment of an 8086 assembly program. This includes using DB (Define Byte), DW (Define Word), and DD (Define Doubleword).

Source Code:

.MODEL SMALL

.STACK 100H

.DATA

CR  EQU 13

LF  EQU 10

NUM DB ?

.CODE

STARTPROC PROC

MOV AX, @DATA

MOV DS, AX

MOV AH, 1

INT 21H

MOV NUM, AL

MOV DL, CR

MOV AH, 2

INT 21H

MOV DL, LF

INT 21H

MOV DL, NUM

MOV AH, 2

INT 21H

MOV AH, 4CH

INT 21H

STARTPROC ENDP

END STARTPROC

Output:

Figure 2: Declaring variables

Problem 3: Adding Two Numbers Without Variables in 8086 Processor

Objective: To perform arithmetic addition of two 8-bit numbers using only CPU registers and immediate values (hard-coded numbers), without defining any variables in the data segment.

Source Code:

.MODEL SMALL

.STACK 100H

.DATA

CR  EQU 13

LF  EQU 10

.CODE

STARTPROC PROC

MOV AX, @DATA

MOV DS, AX

MOV AH, 1

INT 21H

SUB AL, '0'

MOV BL, AL

MOV DL, CR

MOV AH, 2

INT 21H

MOV DL, LF

INT 21H

MOV AH, 1

INT 21H

SUB AL, '0'

MOV BH, AL

MOV DL, CR

MOV AH, 2

INT 21H

MOV DL, LF

INT 21H

MOV AL, BL

ADD AL, BH

ADD AL, '0'

MOV DL, AL

MOV AH, 2

INT 21H

MOV AH, 4CH

INT 21H

STARTPROC ENDP

END STARTPROC

Output

Figure 3: Adding two Numbers

Problem 4: Subtracting Two Numbers Without Variables in 8086 Processor

Objective: To perform arithmetic subtraction of two 8-bit numbers using only CPU registers and immediate values, without defining any variables in the data segment.

Source Code:

.MODEL SMALL

.STACK 100H

.DATA

CR  EQU 13

LF  EQU 10

.CODE

STARTPROC PROC

MOV AX, @DATA

MOV DS, AX

MOV AH, 1

INT 21H

SUB AL, '0'

MOV BL, AL

MOV DL, CR

MOV AH, 2

INT 21H

MOV DL, LF

INT 21H

MOV AH, 1

INT 21H

SUB AL, '0'

MOV BH, AL

MOV DL, CR

MOV AH, 2

INT 21H

MOV DL, LF

INT 21H

MOV AL, BL

SUB AL, BH

ADD AL, '0'

MOV DL, AL

MOV AH, 2

INT 21H

MOV AH, 4CH

INT 21H

STARTPROC ENDP

END STARTPROC

Output:

Figure 4: Subtracting two numbers

Conclusion

This lab provided hands-on experience with four fundamental concepts of 8086 assembly programming. We learned:

- Problem 1: How to interact with the user by displaying messages (output) and reading keyboard strokes (input) using DOS INT 21H services.

- Problem 2: The correct syntax for allocating memory for data, defining variables of different sizes (DB, DW, DD), and initializing them in the .DATA segment.

- Problem 3 & 4: How to perform basic arithmetic operations (Addition and Subtraction) using CPU registers (AL, BL) and immediate values. This demonstrates the processor's ability to manipulate data directly without relying on memory variables.