# Project Name: IoT Based Fish Pond Monitoring System (Structured Report)

- Introduction

- IoT based system for monitoring fish pond water quality

- Designed for real-time data collection and remote monitoring

- Targets common problems in Bangladesh aquaculture

  - sudden fish death

  - poor water quality

  - lack of continuous monitoring

# Objectives

- Monitor water parameters continuously

- Reduce fish mortality

- Enable remote monitoring via WiFi

- Provide alert system for dangerous conditions

- Allow future automation (aerator, feeder)

# System Working Principle

- Sensors collect water data

- ESP32 processes the data

- Data sent via WiFi to cloud

- User views data on mobile/web

- Alerts generated if values exceed safe range

# Main Hardware Components:

  - ESP32 Development Board

    - Role: Main controller and WiFi communication

    - Features

      - Built-in WiFi and Bluetooth

      - Dual core processor

      - Low power consumption

    - Price: 430

Reference: https://store.roboticsbd.com/development-boards/948-esp32-esp-32s-30p-nodemcu-development-board-wireless-wifi-robotics-bangladesh.html

# Alternative

    - NodeMCU ESP8266

    - Limitation: Less powerful

    - Price: 350

    - Reference: https://store.roboticsbd.com/development-boards/694-esp8266-ch340-nodemcu-wifi-module-lua-v3-robotics-bangladesh.html

# Temperature Sensor (DS18B20 Waterproof)

    - Role: Measures water temperature

    - Features

      - Waterproof probe

      - High accuracy ±0.5°C

      - Wide range

    - Price: 220

    - Refrence: https://www.daraz.com.bd/products/ds18b20-waterproof-temperature-sensor-i117163000.html

# pH Sensor (PH 4502C)

    - Role: Measures acidity or alkalinity

    - Features

      - Range: 0 to 14 pH

      - Analog output

      - Needs calibration

    - Price: 2900

    - Reference: https://techshopbd.com/product/ph-sensor-with-module

# Alternative

    - Gravity pH sensor

      - Price: 3750

      - More stable

      - Reference: https://www.bdtronics.com/gravity-analog-ph-sensor-meter-kit-v2-compatible-with-3-3v-and-5v-main-controller-for-water-quality-testing-aquaculture.html

# Turbidity Sensor

    - Role: Measures water clarity

    - Features

      - Detects suspended particles

      - Indicates pollution or algae

      - Price: 820 bdt

      - Reference: https://techshopbd.com/product/water-turbidity-sensor-for-arduino

# Alternative

    - turbidity sensor

      - Price: 965

      - Better accuracy

      - Reference: https://robodocbd.com/product/turbidity-sensor-module

# Relay Module

    - Role: Controls external devices

    - Features

      - Switch high voltage devices

    - Module Names:

      - Aerator Or

      - Water pump

    - Price: 400

    - Reference: https://shorturl.at/NnxJU

# Alternative

    - Solid state relay

      - Faster switching

      - Slightly higher cost

# Buzzer and LED

    - Role: Local alert system

    - Features

      - Low power

      - Simple digital output

    - Price: 200

    - References: https://store.roboticsbd.com/tools-accessories/1898-mechanical-buzzer-with-led-indicator-ad16-22sm-robotics-bangladesh.html

# Power System

    - Components

      - 18650 battery

      - TP4056 charging module

    - Features

      - Rechargeable

      - Portable

- Price: 100

- Reference: https://store.roboticsbd.com/battery-charger/1687-37v-18650-rechargeable-battery-solderable-robotics-bangladesh.html

# Alternative

    - Solar system

      - Price: 500 to 700 BDT

      - Best for outdoor

# Supporting Components

    - Breadboard

    - Jumper wires

    - Waterproof box

    - Price: Varies

# Total Cost Estimation

- Basic system cost: 4800 to 5500 BDT

- Cost varies depending on quality and supplier

# Communication System

- Uses ESP32 built-in WiFi

- No extra module needed

# Alternative communication

- GSM SIM800L

  - Price: 350 to 450 BDT

- LoRa module

  - Price: 600 to 900 BDT

# IoT Platforms

- Blynk

  - Easy mobile interface

- ThingSpeak

  - Data visualization

- Firebase

  - Custom app development

# Parameters Monitored

- Water temperature

- pH level

- Turbidity

- Device status

# Smart Features and Improvements

Automation

- Auto aerator control

- Auto water pump control Smart Monitoring

- Mobile notifications

- Real-time dashboard Advanced Features

- Smart fish feeder

- Solar powered system

- SMS alert system

- Multi pond monitoring

- AI based prediction system

# Advantages

- Low cost and affordable

- Real world application in Bangladesh

- Easy to build and expand

- Reduces manual monitoring

- Scalable to commercial level

# Conclusion

- Provides efficient solution for fish farming problems

- Combines IoT and embedded systems effectively

- Suitable for academic project and real life use

- Can evolve into startup level product with upgrades